Name: Prathamesh Gadekar

Roll No: 15

Class: SE(IT)

#include <pic18f4550.h>

#define RELAY_PIN LATAbits.LATA4

void interrupt extint_isr(void) {

unsigned int i;

if(INT1F)   {

INT1F = 0;

INT1IE = 0;

RELAY_PIN = ~RELAY_PIN;

for(i=0; i<10000; i++);      //small delay for debouncing

INT1IE = 1;

}

}

int main()

{

ADCON1 = 0x0F;          //set pins as Digital

TRISAbits.TRISA4 = 0;   //set relay pin RA4 as output

TRISBbits.TRISB1 = 1;   //Interrupt pin as input

RELAY_PIN = 1;

INT1IE  =   1;                      //Enable external interrupt INT1

INTEDG1 =   0;                      //Interrupt on falling edge

GIE     =   1;                      // Enable global interrupt

while(1);

}