Name: Prathamesh Gadekar

Roll No: 15

Class: SE(IT)

#include <pic18f4550.h>		/* Contains PIC18F4550 specifications */

#define Buzzer LATAbits.LATA5  		/* Define buzzer pin */

unsigned int count = 0;

void interrupt Timer1_ISR()

{

if(TMR1IF==1)

{

//TMR1=0xCF2C;

TMR1L = 0x20;

TMR1H = 0xD1;

count ++;

if (count >= 1000) //measure upto 1000 ms i.e. 1 seconds

{

Buzzer = ~Buzzer;  	/* Toggle buzzer pin  */

count = 0;  //reset count

}

TMR1IF = 0; //timer1 overflow flag to 0

}

}

void main()

{

TRISB=0;  			/* Set as output port */

TRISAbits.TRISA5 = 0;       //set buzzer pin RA5 as output

GIE=1;                      /* Enable Global Interrupt */

PEIE=1;                     /* Enable Peripheral Interrupt */

TMR1IE=1;                   /* Enable Timer1 Overflow Interrupt */

TMR1IF=0;

/* Enable 16-bit TMR1 register,no pre-scale,internal clock, timer OFF */

T1CON=0x80;         /*1:8 prescale*/

TMR1L = 0x20;

TMR1H = 0xD1;

TMR1ON=1;		/* Turn ON Timer1 */

while(1);

}